- SAFETY RULES FOR THIS PROCEDURE:

- PURPOSE OF THE PROCEDURE (PLAN): To use ladders in a safe way

- PROCEDURE STEPS (DO - CHECK):

What should you do before using a portable ladder?

- Use a ladder designed for your task. Consider the strength, type, length and the Canadian Standards Association (CSA) approval.

- Read and follow all the labels and markings on the ladder.

- Make sure the load rating is indicated on the ladder and is appropriate for the user, equipment, and task.

- Inspect the ladder before and after each use.

- Reject and tag any ladders that have defects. Have faulty ladders repaired or thrown out.

- Get help when handling a heavy or long ladder.

- Keep ladders away from power wires, and if electricity is a hazard, a fibreglass ladder should be used.

- Tie off extensions or straight ladders at the top and secure bottom to prevent them from slipping.

- Set up barricades and warning signs when using a ladder in a doorway or passageway.

- Before mounting a ladder, clean the boot soles if they are muddy or slippery. Avoid climbing with wet soles. Ensure that footwear is in good condition.

- Face the ladder when going up or down and when working from it.

- Keep the centre of your body within the middle of the side rails.

- Consider having another worker hold and support the base of the ladder when needed

What should you avoid when using a portable ladder?

- Do not use a ladder in a horizontal position as a scaffold plank or runway.

- Do not carry objects in your hands while on a ladder. Hoist materials or attach tools to a belt.

- Do not work from the top three rungs. The higher a person goes on a ladder, the greater the possibility that the ladder will slip out at the base.

- Do not use items such as a chair, barrel or box as a makeshift ladder.

- Do not use a portable ladder when other types of equipment and safe means of access are available. Replace a ladder with a fixed stairway or scaffold.

- Do not join two short ladders to make a longer ladder. Side rails are not strong enough to support the extra load.

- Do not paint wooden ladders. The paint may hide defects. Wood preservatives or clear coatings may be used.

How should you set up the ladder?

- Make sure to set up the ladder at the required angle, using the 4-to-1 Rule: For every 4 feet (1.2 metres) up, place the base of your ladder 1 foot (0.3 metres) from the wall or upper support it rests against. This placement will set up the ladder with a pitch of 75 degrees, creating the ideal angle for climber balance, ladder strength, and helps prevent the ladder from slipping.

- Make sure always to follow the instructions provided by the manufacturer.

- Extend the ladder at least 1 m (3 ft) above the landing platform or the point of support.

- Place the ladder on a firm, level footing. Use a ladder with slip-resistant feet or secure blocking. Brace or tie the bottom of the ladder.

- Rest both side rails on the top support and secure the ladder to prevent slipping.

What should you know about climbing portable ladders?

- Check for overhead power lines and other hazards before setting up a ladder.

- Clear the area around the base and top of the ladder of debris, tools and other objects.

- Wear a safety harness and tie the lanyard off to a proper anchor (e.g., designed fixed support, temporary fixed support, or existing structural feature or equipment) when working 3 m (10 ft) or more off the ground or when working with both hands. Make sure that you have been trained on how to use fall protection devices safely.

- Ensure that only one person is on a single-width ladder. Only one person is allowed on each side of a double-width ladder.

- Maintain three-point contact by keeping two hands and one foot or two feet and one hand on the ladder at all times.

- Grasp the rungs when climbing a ladder, not the side rails. If your foot slips on a ladder, holding onto rungs is easier than holding onto the side rails.

- Wear protective footwear with slip-resistant soles and heels.

- Ensure that all electrical equipment used during ladder work is in good condition and properly grounded.

- Rest frequently to avoid arm fatigue and disorientation when the work requires you to look up and reach above your head.

- Drape your arms over a rung and rest your head against another rung or side rail if you become dizzy or panicky. Climb down slowly.

What should you avoid when climbing portable ladders?

- Do not use ladders when a safe means of access is available and practical.

- Do not use a ladder in passageways, doorways, driveways, or other locations where a person or vehicle can hit it. Set up suitable barricades or lock the doors shut.

- Do not place a ladder against flexible or moveable surfaces.

- Do not straddle the space between a ladder and another object.

- Do not erect ladders on boxes, carts, tables, scaffolds, or other unstable surfaces.

- Do not use ladders on ice, mud, or other slippery surfaces.

- Do not stand a ladder on any of its rungs. Ladders must rest on both side rails.

- Do not allow anyone to stand under a ladder.

- Do not support ladders on their rungs.

- Do not overreach from a ladder; move as required.

- Do not use any type of ladder near overhead power lines unless the safe distance is maintained. Check with your local jurisdiction as this distance may be regulated in your area.

- REFERENCE DOCUMENTS:

OCCUPATIONAL HEALTH AND SAFETY REGULATIONS

| Author | Description of version changes (Control of changes): | Revision Control: | Date: |
| --- | --- | --- | --- |
| HSE | Revision 1 | 1 | Jan 21 2025 |